# USART2 Verification Plan

RISC-V VP++ | SystemC USART2 Peripheral | Rev 1.0

# 1. Introduction

This document describes the verification plan for the USART2 peripheral model implemented in the riscv-vp-plusplus SystemC virtual prototype. The USART2 is a synchronous/asynchronous serial interface peripheral modelled as a SystemC sc_module with TLM-2.0 target socket, verified through bare-metal RISC-V firmware running on the RV32 instruction-set simulator (ISS).

The verification approach exercises the VP as an integrated SoC: the RISC-V core, PLIC interrupt controller, CLINT, system bus, and both USART instances (A and B) wired back-to-back via sc_fifo. No component is stubbed; all interactions traverse the real TLM bus path.

# 2. Verification Scope

In-scope:

Register access: CON, TBUF, RBUF, STATUS (read/write/W1C semantics)

Interrupt sources: TBIR, TIR, RIR, EIR — generation, ordering, gating

Data integrity: all 256 byte values, boundary patterns, A↔B paths

Error conditions: overrun (OE/EIR), OEN gating, recovery after overrun

Timing: TBIR immediacy, TIR 2µs delay, RIR-after-TBIR ordering

Negative: unimplemented registers, unknown offsets, reserved bit writes

Stress: 100+ byte sustained transfers, overrun cycles, memory leak target

Out-of-scope (not implemented in VP model):

BG register (baud generator) — no-op in VP; logged as finding BUG-002

FDR register (fractional divider) — no-op in VP; logged as finding BUG-003

CON.LB loopback routing — stored but not functional; logged as BUG-004

Synchronous mode (Mode 0) — VP uses fixed 2µs frame delay

Parity generation/checking — VP delivers raw bytes

Frame error (FE) detection — not applicable to VP model

# 3. USART Architecture

## 3.1 VP Model Summary

The Usart2 SystemC module (vp/src/platform/usart2test/usart2.h) implements a simplified USART model for interrupt validation. It contains:

TLM-2.0 simple_target_socket for register access via bus

SC_THREAD rx_thread: blocks on sc_fifo_in<uint8_t>, delivers to RBUF

SC_METHOD tir_method: fires STATUS_TIR on m_frame_done_ev (2µs after TBUF write)

SC_METHOD tx_deliver_method: forwards byte to sc_fifo_out (co-fires with tir_method)

Interrupt gateway pointer to PLIC; calls gateway_trigger_interrupt(irq_id)

5 SC signals per instance for VCD tracing (txd, rxd, tbir, tir, rir, eir, irq)

## 3.2 Register Map

| Offset | Name | Access | VP Status | Description |
| --- | --- | --- | --- | --- |
| 0x00 | CON | R/W | Stored (OEN functional) | Control register |
| 0x04 | TBUF | W | Implemented | TX holding buffer — triggers TBIR+TIR |
| 0x08 | RBUF | R | Implemented | RX holding buffer — drains sc_fifo |
| 0x0C | BG | R/W | No-op | Baud generator — NOT implemented |
| 0x10 | FDR | R/W | No-op | Fractional divider — NOT implemented |
| 0x20 | STATUS | R/W1C | Implemented | Sticky interrupt status |

## 3.3 Hardware Signal Reference

The following tables document every hardware input, output, and bidirectional signal defined in the USART Controller Specification (Rev 1.0, Table 1) and cross-reference each signal against the VP model implementation in usart2.h. Three implementation states are used:

| IMPLEMENTED | Signal exists and is functionally correct in the VP model. |
| --- | --- |
| ABSTRACTED | Signal exists but is modelled at a higher level of abstraction (e.g. byte-level sc_fifo instead of bit-serial TXD pin). |
| NOT IMPLEMENTED | Signal is defined in the spec but entirely absent from the VP model. A bug entry (BUG-xxx) is raised. |
|  | (status indicator legend above — used throughout this section) |

Table 1 — Physical Pins  (Spec §3, Table 1 — Port List)

| Pin | Dir | Spec Description | VP Implementation | Status |
| --- | --- | --- | --- | --- |
| TXD | Output | Async: serial data out, LSB first. Sync (Mode 0): clock output from ÷4 sampler; idle = HIGH. | Replaced by sc_fifo_out<uint8_t> tx_port. Bytes transferred whole; no bit-serial shifting. Idle HIGH tracked by sig_txd_line. | Abstracted |
| RXD | Bidir | Async: serial data in; idle = HIGH. Sync: CON.REN=0 → output (TX turn), CON.REN=1 → input (RX turn). | Replaced by sc_fifo_in<uint8_t> rx_port. Direction switching (CON.REN) is stored in m_con but has no routing effect on sc_fifo. | Abstracted |
| CLK | Input | Peripheral clock f_PERIPH; gated off when CON.R=0. | No clock signal in the SystemC model. CON.R is stored but never checked. Module runs freely. Baud rate not derived from any clock. | Not Implemented |
| RES | Input | Hardware reset — clears ALL registers and internal state. | No sc_in<bool> rst port. Offset 0x14 (SWRST) falls into default: no-op in b_transport. State is never externally reset. -> BUG-007 | Not Implemented |

Table 2 — Interrupt Output Signals  (Spec §4.7)

| Signal | Dir | Spec Assertion Condition | VP Implementation | Status |
| --- | --- | --- | --- | --- |
| TBIR | Output | TBUF contents transferred to TSR — holding buffer slot free. Fires earlier than TIR. Pulse = 2 x f_PERIPH. | Set synchronously inside b_transport on OFF_TBUF write (m_status ¦= STATUS_TBIR; fire_irq()). Fires in same TLM transaction as TBUF write. Reflected in sig_tbir. | Implemented |
| TIR | Output | Last stop bit begins shifting onto TXD — TSR empty after stop period. Pulse = 2 x f_PERIPH. | Scheduled via m_frame_done_ev.notify(2µs) at TBUF write time. Fires in tir_method() SC_METHOD 2µs later. Reflected in sig_tir. | Implemented |
| RIR | Output | Complete received frame transferred from RSR to RBUF — data ready. Pulse = 2 x f_PERIPH. | Fires inside rx_thread() when sc_fifo_in::read() delivers a byte and m_rbuf_full==false (m_status ¦= STATUS_RIR; fire_irq()). Arrives ~2µs after transmitter TBUF write. Reflected in sig_rir. | Implemented |
| EIR | Output | Simultaneous with RIR when OE/FE/PE is set and corresponding enable bit (OEN/FEN/PEN) is 1. Pulse = 2 x f_PERIPH. | Fires in rx_thread() when byte arrives but m_rbuf_full==true AND (m_con & CON_OEN) is set. OE path only — FE and PE never fire (no framing/parity logic). Reflected in sig_eir. | Partial (OE only) |
| IRQ (combined) | Output | Not a separate spec pin. Aggregate interrupt to CPU. | sig_irq is HIGH whenever any STATUS bit is set (STATUS & 0xF). Delivered to PLIC via gateway_trigger_interrupt(irq_id). USART_A uses IRQ 1, USART_B uses IRQ 2. | Implemented |

Table 3 — Error Flag Signals  (Spec §4.6)

| Flag | Spec Condition | Clear Procedure | VP Status | Notes |
| --- | --- | --- | --- | --- |
| OE — Overrun Error | RSR completes a new frame while RBUF still holds unread data. New frame discarded. Applies to all modes. | Read RBUF, or write 0 to CON.OE. | Implemented | Detected in rx_thread when m_rbuf_full==true. EIR fires if CON.OEN=1. New byte silently discarded. Cleared when firmware reads RBUF (m_rbuf_full set to false). |
| FE — Framing Error | Expected stop bit(s) sampled as logic 0 — baud mismatch, break, or noise. Async only. | Automatic on next valid start bit. | Not Implemented | No start/stop bit serialization in VP. Bytes transferred as whole octets via sc_fifo. FE can never be generated. CON.FEN is stored but ignored. |
| PE — Parity Error | Received parity bit does not match parity computed over data bits using CON.ODD setting. Modes 4/5 only. | Read RBUF. | Not Implemented | No parity computation in VP. CON.PEN and CON.ODD stored but ignored. RBUF[0] parity bit placement (spec §6.5) not modelled. |

Table 4 — VCD Trace Signals  (VP-specific, no physical counterpart in spec)

| Signal | SC Type | Driven By | Meaning |
| --- | --- | --- | --- |
| sig_txd | sc_uint<8> | b_transport (TBUF write) | Data byte being transmitted. Updated each time TBUF is written. |
| sig_txd_parity | bool | b_transport (TBUF write) | Even parity of sig_txd (XOR of all 8 data bits). |
| sig_txd_line | bool | b_transport + tir_method | LOW = TX frame in progress. HIGH = TX line idle. Matches TXD idle-HIGH convention. |
| sig_rxd | sc_uint<8> | rx_thread | Data byte just received from sc_fifo. Updated each time a byte arrives. |
| sig_rxd_parity | bool | rx_thread | Even parity of sig_rxd. |
| sig_rxd_line | bool | rx_thread + b_transport (RBUF read) | LOW = received byte pending in RBUF. HIGH = RX buffer drained. |
| sig_tbir | bool | sync_irq_trace() | HIGH while STATUS[TBIR] (bit 0) is set. Mirrors interrupt state. |
| sig_tir | bool | sync_irq_trace() | HIGH while STATUS[TIR] (bit 1) is set. Mirrors interrupt state. |
| sig_rir | bool | sync_irq_trace() | HIGH while STATUS[RIR] (bit 2) is set. Mirrors interrupt state. |
| sig_eir | bool | sync_irq_trace() | HIGH while STATUS[EIR] (bit 3) is set. Mirrors interrupt state. |
| sig_irq | bool | sync_irq_trace() | HIGH while any STATUS bit is set. Aggregate interrupt indicator. |

Table 5 — SystemC Interface Ports  (VP internal — replace physical pins in simulation)

| Port | SC Type | Direction | Connected To | Replaces Spec Pin |
| --- | --- | --- | --- | --- |
| tsock | simple_target_socket | Input | System bus (SimpleBus port mapping 0x09002000/0x09003000) | CPU bus interface — carries all register R/W TLM transactions. |
| tx_port | sc_fifo_out<uint8_t> | Output | fifo_a2b (USART_A) or fifo_b2a (USART_B) — capacity 16 bytes | TXD — abstracted to byte-level delivery instead of bit-serial. |
| rx_port | sc_fifo_in<uint8_t> | Input | fifo_b2a (USART_A) or fifo_a2b (USART_B) — capacity 16 bytes | RXD — abstracted to byte-level reception. |
| plic | interrupt_gateway* | Output (pointer) | FE310_PLIC instance; set in sc_main before sc_start() | TBIR, TIR, RIR, EIR — all four multiplexed onto one PLIC IRQ line. |
| irq_id | uint32_t | Configuration | Set in sc_main: usart_a.irq_id=1, usart_b.irq_id=2 | Identifies which PLIC IRQ line to assert for each instance. |

Table 6 — Spec vs VP Implementation Summary

| Spec Signal | Spec Type | Implemented? | Gap / Bug Reference |
| --- | --- | --- | --- |
| TXD | Physical Output | Abstracted | Byte-level sc_fifo; no bit serialization, no start/stop bits. |
| RXD | Physical Bidir | Abstracted | Byte-level sc_fifo; no oversampling, no glitch rejection, no direction switch. |
| CLK (f_PERIPH) | Physical Input | Not Implemented | No clock port; module runs without a clock; CON.R (run bit) ignored. |
| RES (reset) | Physical Input | Not Implemented | BUG-007 — no reset pin and no SWRST register. State is never reset mid-simulation. |
| TBIR | Interrupt Output | Implemented | Fires synchronously on TBUF write via PLIC gateway_trigger_interrupt(). |
| TIR | Interrupt Output | Implemented | Fires 2µs after TBUF write via sc_event::notify(2µs). |
| RIR | Interrupt Output | Implemented | Fires when rx_thread delivers byte to m_rbuf. |
| EIR | Interrupt Output | Partial (OE only) | OE path implemented; FE and PE never generated (no framing/parity logic). |
| OE error | Internal Flag | Implemented | Detected when m_rbuf_full==true on new byte arrival. Gated by CON.OEN. |
| FE error | Internal Flag | Not Implemented | Requires bit-serial framing — not modelled in VP. |
| PE error | Internal Flag | Not Implemented | Requires parity computation — not modelled in VP. CON.ODD/PEN stored only. |
| Baud rate (BG/FDR) | Internal | Not Implemented | BUG-002/003 — BG and FDR registers are no-ops; frame delay is fixed at 2µs. |
| CON.LB loopback | Mode bit | Not Implemented | BUG-004 — CON.LB stored in m_con; sc_fifo routing fixed at elaboration. |
| Sync mode (Mode 0) | Operating mode | Not Implemented | No ÷4 clock output on TXD, no half-duplex direction control. |
| 9-bit mode (Mode 3) | Operating mode | Not Implemented | TX9/RX9 bits stored in CON but not forwarded in byte transfers. |
| Parity modes (4/5) | Operating mode | Not Implemented | RBUF[0] parity bit placement (spec §6.5) not modelled. |

# 4. Register Summary

All registers are 32-bit word-aligned. The VP model stores the full 32-bit CON value (including reserved bits [31:16]). Only CON.OEN (bit 6) has a functional effect on interrupt generation. STATUS is a sticky write-1-to-clear register.

| Field | Bits | Access | VP Functional | Note |
| --- | --- | --- | --- | --- |
| CON.M[2:0] | [2:0] | R/W | Stored only | Mode select — model ignores |
| CON.REN | [3] | R/W | Stored only | Receiver enable |
| CON.OEN | [6] | R/W | Yes | Gates EIR on overrun |
| CON.FEN | [7] | R/W | Stored only | Framing error enable |
| CON.PEN | [8] | R/W | Stored only | Parity error enable |
| CON.LB | [14] | R/W | Stored only | Loopback — no routing effect |
| CON.R | [15] | R/W | Stored only | Module run bit |
| STATUS.TBIR | [0] | R/W1C | Yes | TX buffer interrupt |
| STATUS.TIR | [1] | R/W1C | Yes | TX complete interrupt |
| STATUS.RIR | [2] | R/W1C | Yes | RX data ready |
| STATUS.EIR | [3] | R/W1C | Yes | Error interrupt (gated by OEN) |

# 5. Verification Environment

## 5.1 Complete Ubuntu Setup Guide

STEP 1 — Install Ubuntu 24.04 LTS

Download Ubuntu 24.04 LTS ISO from ubuntu.com/download/desktop. For dual-boot with Windows: shrink Windows partition via Disk Management, create a ≥50 GB unallocated partition, boot from USB, select 'Install alongside Windows'. Enable Secure Boot or disable it in BIOS/UEFI for unsigned kernel modules.

STEP 2 — Install system dependencies

sudo apt-get update && sudo apt-get install -y \
  build-essential cmake git python3-pip \
  libboost-iostreams-dev libboost-program-options-dev libboost-log-dev \
  nlohmann-json3-dev libvncserver-dev gcc-riscv64-linux-gnu \
  valgrind gtkwave code

STEP 3 — Set up RISC-V cross-compiler symlinks

mkdir -p ~/.local/bin
for t in gcc as ld ar objcopy objdump nm ranlib readelf strip size; do
    ln -sf /usr/bin/riscv64-linux-gnu-$t ~/.local/bin/riscv32-unknown-elf-$t
done
echo 'export PATH=$HOME/.local/bin:$PATH' >> ~/.bashrc && source ~/.bashrc

STEP 4 — Install Claude Code in VS Code

Install VS Code: sudo snap install --classic code. Open VS Code, press Ctrl+Shift+X, search 'Claude Code' and install. Open a terminal inside VS Code and run: claude — authenticate with your Anthropic account. The Claude Code extension connects to the CLI automatically.

STEP 5 — Python packages for XLSX/DOCX generation

pip install openpyxl python-docx

## 5.2 VP Workspace Build

git clone <repo_url> riscv-vp-plusplus
cd riscv-vp-plusplus
git submodule update --init -- vp/src/vendor/systemc
cmake -B vp/build -S vp -DCMAKE_BUILD_TYPE=Release
cmake --build vp/build --target usart2test-vp -j$(nproc)
# Binary: vp/build/bin/usart2test-vp

# 6. Test Strategy

Each test category is an independent RISC-V ELF firmware image that runs on the usart2test-vp binary. Tests communicate with the VP via memory-mapped registers and receive interrupts through the PLIC/ISS trap mechanism. All test results are printed to the console UART and captured to timestamped log files.

| Strategy | Description |
| --- | --- |
| White-box | Tests designed with knowledge of VP model internals (m_rbuf, m_con, etc.) |
| Integration | Full SoC path: ISS → TLM bus → USART → sc_fifo → USART → PLIC → ISS |
| Functional | Verify interrupt generation, data integrity, error conditions |
| Regression | Automated via run_all.sh; logs archived with timestamps |
| Valgrind | memory_leak test ELF run under valgrind for VP-side leak detection |
| Coverage | Functional ≥95%, Error 100%, Mode 100% target |

# 7. Test Categories

| Category | Test File | Test IDs | Count | Coverage Focus |
| --- | --- | --- | --- | --- |
| Reset | test_reset.c | RST-001..005 | 5 | Register reset values, state recovery |
| R/W | test_rw.c | RW-001..007 | 7 | Register read/write semantics |
| Functional | test_functional.c | FN-001..005 | 5 | All 4 interrupt sources |
| Clock | test_clock.c | CLK-001..004 | 4 | Timing and interrupt ordering |
| Loopback | test_loopback.c | LB-001..004 | 4 | A↔B data path integrity |
| Advanced | test_advanced.c | ADV-001..006 | 6 | OEN gating, recovery, bidirectional |
| Performance | test_performance.c | PERF-001..004 | 4 | Throughput and latency |
| Stress | test_stress.c | STR-001..003 | 3 | Sustained transfer stability |
| Negative | test_negative.c | NEG-001..007 | 7 | Error injection, invalid accesses |
| Datatype | test_datatype.c | DT-001..003 | 3 | Full 256-value data coverage |
| MemoryLeak | test_memory_leak.c | MEM-001..003 | 3 | Valgrind stability target |

# 8. Detailed Test Cases

Refer to USART_TestPlan.xlsx for the complete test case matrix with objectives, expected results, register coverage, and pass/fail status. Key test cases are summarised below:

| ID | Name | Stimulus | Expected | Pass Criteria |
| --- | --- | --- | --- | --- |
| FN-001 | A→B 0x55 | UA_TBUF=0x55 | TBIR_A+RIR_B, RBUF_B=0x55 | Both in log, data match |
| FN-003 | Overrun EIR | Fill RBUF_B; UA_TBUF=0x22 | EIR_B with OEN=1 | EIR_B in log |
| FN-005 | TIR separate | UA_TBUF=0x5A | TBIR_A then TIR_A 2µs later | TIR after TBIR in log |
| NEG-005 | No EIR OEN=0 | OEN=0; overrun | EIR absent | EIR not in log |
| DT-001 | 256 sweep | 0x00..0xFF A→B | 0 errors | All RBUF_B match sent |
| STR-001 | 100 bytes | Walk pattern A→B | 0 errors | All 100 bytes correct |

# 9. Coverage Strategy

Coverage is tracked across eight dimensions. All coverage data is captured in coverage_summary.xlsx.

| Type | Target | Method | Current |
| --- | --- | --- | --- |
| Functional | ≥95% | Test execution vs feature matrix | ~97% |
| Register | 100% | R/W/W1C test for each register | 86% (BG/FDR not impl.) |
| Interrupt | 100% | TBIR/TIR/RIR/EIR all exercised | 100% |
| Error | 100% | OE/EIR with/without OEN | 100% |
| Data | 100% | All 256 byte values (DT-001) | 100% |
| Negative | 100% | Invalid registers, bad patterns | 100% |
| Stress | >50 transfers | STR-001/002/003 | 100% |
| Timing | TBIR<TIR, RIR>TBIR | CLK tests | 100% |

# 10. Build and Run Guide

## 10.1 Build All Test ELFs

cd sw/usart2_verif
mkdir -p build && cd build
cmake -DCMAKE_BUILD_TYPE=Release ..
make -j$(nproc) install
# ELFs in build/bin/

## 10.2 Full Regression

cd sw/usart2_verif
./scripts/run_all.sh
# Logs: logs/regression/<TIMESTAMP>/

## 10.3 Single Test

./scripts/run_single_test.sh functional
./scripts/run_single_test.sh stress --vcd
./scripts/run_single_test.sh reset --no-build

## 10.4 Valgrind Memory Leak Check

valgrind --leak-check=full --error-exitcode=1 \
  ./vp/build/bin/usart2test-vp \
  ./sw/usart2_verif/build/bin/test_memory_leak.elf

## 10.5 Generate Documentation

cd sw/usart2_verif
python3 docs/generate_xlsx.py   # USART_TestPlan.xlsx, errata.xlsx, coverage.xlsx
python3 docs/generate_docx.py   # USART_Verification_Plan.docx

# 11. Log Archival

Each run_all.sh invocation creates a timestamped directory under logs/regression/. Individual test logs are named <test_name>.log. A SUMMARY.txt aggregates all PASS/FAIL/FINDING lines. Failing tests are automatically copied to logs/failures/. archive_logs.sh compresses old runs to logs/archived/ keeping the 5 most recent.

| Directory | Contents |
| --- | --- |
| logs/regression/<TS>/ | One .log per test + SUMMARY.txt |
| logs/failures/ | Copy of logs for failed tests (named TS_test_name.log) |
| logs/archived/ | Compressed .tar.gz of older regression runs |

# 12. Errata Handling

Tests that expose spec/model divergences print a [FINDING] line to the log. Findings are collected in usart_errata.xlsx. Each errata entry includes: Bug ID, Test ID, expected spec behaviour, actual VP model behaviour, root cause, suggested fix, severity, status, and owner.

| Severity | Criteria | Action |
| --- | --- | --- |
| High | Functional mismatch affecting correctness | Fix in VP model ASAP |
| Medium | Missing feature; workaround possible | Schedule for next sprint |
| Low | Reserved bit masking; cosmetic | Fix when convenient |
| Info | Documented difference; no fix needed | Log and close |

# 13. Coverage Summary

| Category | Tests | Passed | Findings | Coverage % |
| --- | --- | --- | --- | --- |
| Register Coverage | 14 | 12 | 2 | 86% |
| Functional Coverage | 9 | 9 | 0 | 100% |
| Interrupt Coverage | 5 | 5 | 0 | 100% |
| Error Coverage | 6 | 6 | 0 | 100% |
| Timing Coverage | 4 | 4 | 0 | 100% |
| Data Coverage | 5 | 5 | 0 | 100% |
| Stress Coverage | 5 | 5 | 0 | 100% |
| Performance Coverage | 4 | 4 | 0 | 100% |
| Negative Coverage | 7 | 7 | 0 | 100% |
| TOTAL | 59 | 57 | 2 | 97% |

# 14. Conclusion

The USART2 VP model has been verified through 59 test cases across 11 categories. 57 tests pass outright. 6 findings were identified — 4 are spec-vs-model gaps (BG, FDR, CON.LB not implemented; reserved bits not masked) and 2 are documented VP-specific behaviours (RBUF stale re-read). Error interrupt gating via CON.OEN is correctly implemented. All 256 data values pass integrity checks across both A→B and B→A paths. The VP is suitable for firmware interrupt-driven USART development with the noted limitations on baud-rate simulation and loopback mode.